Edge Computing Lab

Class: TY-AIEC

School of Computing, MIT Art Design Technology University

Academic Year: 2024-25

Experiment No. 3

Title

DHT11 Sensor and Alert System using Blynk IoT

Objective:

The goal of this project is to create a system with a DHT11 sensor interfaced with a Raspberry Pi that  monitors humidity levels and sends alerts via the Blynk IoT platform when humidity exceeds 70%.

Materials:

- Raspberry Pi (any model with GPIO pins)

- DHT11 Temperature and Humidity Sensor

- Breadboard and jumper wires

- Resistors (typically 10kΩ for DHT11 pull-up)

- Blynk Mobile App

- Internet connection

Procedure:

Task 1: Connect a DHT11 to the Raspberry Pi

1. Initial Setup: Ensure your Raspberry Pi is set up with the latest version of Raspbian OS and is  connected to the internet.

2. Wiring: Connect the DHT11 sensor to the Raspberry Pi GPIO pins.

- VCC pin to a 5V pin on the Raspberry Pi.

- Data pin to a GPIO pin (e.g., GPIO4).

- GND pin to a ground pin on the Raspberry Pi.

- Place a 10kΩ resistor between VCC and the Data pin (this acts as a pull-up resistor).

Task 2: Program the Raspberry Pi

1. Install Libraries: Install the DHT11 Python library by running `sudo pip install dht11` in the  terminal.

2. Coding:

11

Lab Manual

- Write a Python script that reads humidity and temperature from the DHT11 sensor.  - Include a conditional statement to check if the humidity is greater than 70%.  - If the condition is true, use the Blynk library to send a notification.

Task 3: Configure the Blynk IoT

1. Blynk App Setup: Download and install the Blynk app on your mobile device or desktop. 2. Create a New Project:

- Open the app and create a new project.

- Select the device as Raspberry Pi and the connection type as Wi-Fi.

- An authentication token will be sent to your email, which will be used in your Python script.

Task 4: Generate the GUI on Mobile / Desktop

1. Adding Widgets:

12

Lab Manual

- In the Blynk app project, add a Gauge widget for displaying humidity.

- Add a Notification widget that will be used to send alerts.

- Add the DataStream

Task 5: Apply Analytics for Alert Generations in Blynk IoT

1. Script Enhancement:

- Modify the Python script to send data to Blynk using the Virtual Pins.

- Use Blynk’s `eventor` feature to set up the logic for alert generation based on the humidity value. 2. Data Logging: Use Blynk’s Super-Chart widget to log and display humidity data over time. Execution:

1. Run the Python script on the Raspberry Pi.

2. Ensure that the script is reading the DHT11 sensor data correctly.

3. Monitor the Blynk app dashboard for real-time data.

13

Lab Manual

Code:

import time

import Adafruit_DHT

import BlynkLib

# Replace with your Blynk Auth Token

BLYNK_AUTH_TOKEN = '_XnsCg95jOshh7O2jiBuMq7oWbVA6iCr'

# Set the sensor type and the GPIO pin

DHT_SENSOR = Adafruit_DHT.DHT11

DHT_PIN = 4  # GPIO pin number where the sensor's data pin is connected

# Initialize Blynk

blynk = BlynkLib.Blynk(BLYNK_AUTH_TOKEN)

# Function to read DHT11 data and send it to Blynk

@blynk.on("connected")

def read_and_send_data():

humidity, temperature = Adafruit_DHT.read(DHT_SENSOR, DHT_PIN)

if humidity is not None and temperature is not None:

print(f'Temperature: {temperature}C, Humidity: {humidity}%')

blynk.virtual_write(1, temperature)  # Send temperature to virtual pin V1

blynk.virtual_write(0, humidity) 	# Send humidity to virtual pin V2

else:

print('Failed to retrieve data from sensor')

# Main loop

while True:

read_and_send_data()  # Read and send data

blynk.run()  # Keep Blynk connection alive

time.sleep(1)  # Wait for 10 seconds before reading again

Results:

- The system should accurately read the humidity levels from the DHT11 sensor. - The Blynk app should display real-time humidity data.

- Upon reaching the 70% humidity threshold, the system should send a notification alert. Discussion:

- Potential issues could include inaccurate readings from the DHT11 sensor, which may require  calibration.

- Network instability could affect the performance of the Blynk app notifications.

Conclusion:

The system successfully integrates a DHT11 sensor with a Raspberry Pi to monitor humidity levels and  uses the Blynk IoT platform to send alerts when thresholds are exceeded, demonstrating the viability of  IoT for home automation and monitoring tasks.